# IPV6 Address Lab

Topology

IP address details

R1 GE0/1- 2001:db8:acad:1::1/64

R1 Ge0/0-2001:db8:acad:2::1/64

1.Configure R1 LAN interface to the switch

R1(config)# interface GigabitEthernet0/1

ipv6 address 2001:db8:acad:1::1/64

ipv6 enable

no shutdown

exit

2. Configure R1 interface to R2

R1(config)# interface GigabitEthernet0/0

ipv6 address 2001:db8:acad:2::1/64

ipv6 enable

no shutdown

exit

3.Configure R2 as a SLAAC Client

R2> enable

R2# configure terminal

R2(config)# interface GigabitEthernet0/0

ipv6 enable

ipv6 address autoconfig

no shutdown

Enable IPv6 Routing on R1

R1> enable

R1# configure terminal

R1(config)# ipv6 unicast-routing

Verification command

R2# show ipv6 interface brief

NOTE: In IPv6 networking, RA stands for Router Advertisement.

It is a multicast ICMPv6 message (Type 134) sent by a router to announce its presence on a local network. RAs provide connected devices with critical configuration information, including the network prefix, the default gateway address, and specific flags that instruct the device on whether to use SLAAC or DHCPv6 to configure its own IP address.